|  | NE40E-M2 V800R012SPH330 Patch Release Notes |  |  |  |
| --- | --- | --- | --- | --- |
|  | Issue | 01 |  |  |
|  | Date | 2023-04-07 |  |  |
|  | HUAWEI TECHNOLOGIES CO., LTD. |  |  |  |

Copyright © Huawei Technologies Co., Ltd. 2023. All rights reserved. No part of this document may be reproduced or transmitted in any form or by any means without prior written consent of Huawei Technologies Co., Ltd. Trademarks and Permissions and other Huawei trademarks are trademarks of Huawei Technologies Co., Ltd. All other trademarks and trade names mentioned in this document are the property of their respective holders. Notice The purchased products, services and features are stipulated by the contract made between Huawei and the customer. All or part of the products, services and features described in this document may not be within the purchase scope or the usage scope. Unless otherwise specified in the contract, all statements, information, and recommendations in this document are provided "AS IS" without warranties, guarantees or representations of any kind, either express or implied. The information in this document is subject to change without notice. Every effort has been made in the preparation of this document to ensure accuracy of the contents, but all statements, information, and recommendations in this document do not constitute a warranty of any kind, express or implied.

| Huawei Technologies Co., Ltd. |  |
| --- | --- |
| Address: | Huawei Industrial Base Bantian, Longgang Shenzhen 518129 People's Republic of China |
| Website: | https://www.huawei.com |
| Email: | support@huawei.com |

Contents

1 Patch Package Information	1

1.1 Product Name	1

1.2 Product Model	1

1.3 Patch Number	1

1.4 Release Date	1

1.5 Patch File Description	2

1.6 Product Version and Required Patches	2

1.7 Relationship Between Patches	3

1.7.1 V800R012C10SPC300	3

1.8 Impact of Patch Installation	4

1.9 Precautions	4

1.10 Naming Rules for Patches	5

2 Resolved Issues	7

2.1 How to Obtain Issues Resolved	7

3 Installing the Patch	9

3.1 Tool-based Patch Upgrade (Recommended)	9

3.2 Installing the Patch	9

3.2.1 Preparing to Install	9

3.2.2 Installing the Patch	11

3.3 Verifying Patch Installation	11

3.3.1 NE40E-M2 V800R012C10SPC300	12

3.4 Follow-up Procedures After Patch Installation	12

4 Rollback Procedure	13

4.1 How to Roll Back to the Previous State	13

5 Solutions to Common Problems During Patch Installation	14

5.1 A Message "Error: The system file should be in root directory." Is Displayed After the patch load all run Command Is Run to Load a Patch File.	14

5.2 A Message "Error: The command is invalid in the current patch status." Is Displayed After the patch load all run Command Is Run to Load a Patch File.	14

5.3 A Message "Error: The patch is incompatible with the existing patch." Is Displayed After the patch load all run Command Is Run to Load a Patch File.	15

5.4 How Can I Set a Patch as That to Be Loaded at the Next Startup?	15

A Vulnerabilities Fixed	16

# Patch Package Information

## Product Name

HUAWEI NetEngine40E-M2 Series Universal Service Router

## Product Model

NE40E-M2E,NE40E-M2F,NE40E-M2H,NE40E-M2K, NE40E-M2K-B

## Patch Number

NE40E-M2 V800R012SPH330

## Release Date

| Released Patch Version | Patch Release Date |
| --- | --- |
| V800R012SPH330 | 2023-04-07 |
| V800R012SPH2b0 | 2022-11-30 |
| V800R012SPH280 | 2022-08-30 |
| V800R012SPH073 | 2022-05-30 |
| V800R012SPH072 | 2022-04-28 |
| V800R012SPH071 | 2022-03-23 |
| V800R012SPH070 | 2022-03-03 |
| V800R012SPH060 | 2021-12-21 |
| V800R012SPH059 | 2021-11-30 |
| V800R012SPH057 | 2021-10-15 |
| V800R012SPH055 | 2021-09-14 |
| V800R012SPH053 | 2021-08-19 |
| V800R012SPH052 | 2021-08-11 |
| V800R012SPH039 | 2021-06-30 |
| V800R012SPH036 | 2021-06-14 |
| V800R012SPH032 | 2021-04-28 |
| V800R012SPH031 | 2021-04-19 |
| V800R012SPH028 | 2021-03-10 |
| V800R012SPH023 | 2020-12-31 |
| V800R012SPH019 | 2020-10-31 |
| V800R012SPH017 | 2020-10-19 |
| V800R012SPH016 | 2020-09-30 |
| V800R012SPH015 | 2020-07-31 |
| V800R012SPH010 | 2020-06-30 |

## Patch File Description

                - Patch file description

| File Name | File Description | File Size |
| --- | --- | --- |
| NE40E-M2V800R012SPH330-C10SPC300.PAT | V800R012SPH330 patch file | 47,283,794 bytes |

## Product Version and Required Patches

| Product Version | Latest Patch Version |
| --- | --- |
| NE40E-M2 V800R012C10SPC300 | NE40E-M2V800R012SPH330-C10SPC300.PAT |

Remarks: The current patch package contains the latest patches for all the product versions. If the latest patch listed in the table or a later patch has been installed on an NE, no more patch needs to be installed.

## Relationship Between Patches

Relationships between patches are classified as function inheritance or incremental relationships.

- Function inheritance relationship: If the latest patch resolves all issues that have been resolved in earlier patches, functions are inherited. Otherwise, functions are not inherited.

- Incremental relationship: If the latest patch can be directly installed on a device with earlier patches installed, the relationship between patches is considered an incremental relationship. Otherwise, the relationship between patches is considered a non-incremental relationship.

Before installing an incremental patch, doe not remove the existing historical patch on a device, which prevents triggering problems that have been resolved using the historical patch.

Before installation a non-incremental patch, note the following issues:

- If a device needs to be restarted or a software package needs to be upgraded, do not remove an existing historical patch from a device. Set the latest patch as the patch to be loaded at the next startup. After the device is restarted, the patch can take effect.

- If services keep running on a device, remove an existing historical patch from a device and then install the latest patch. During the historical patch removal and latest patch installation, problems that have been resolved using the historical patch may be triggered. Evaluate such risk before you operate the patches. Do not perform any operations on the device between uninstalling the earlier patches and installing the latest patch. Otherwise, services may be interrupted. For example, saving configurations causes the commands added in the patches to be lost.

### V800R012C10SPC300

                - Relationships between patches

| Latest Patch | Earlier Patch | Function Inheritance Relationship | Incremental Relationship |
| --- | --- | --- | --- |
| V800R012SPH330 | V800R012SPH2b0 | Inheritance | Incremental |
|  | V800R012SPH280 | Inheritance | Incremental |
|  | V800R012SPH073 | Inheritance | Incremental |
|  | V800R012SPH072 | Inheritance | Incremental |
|  | V800R012SPH071 | Inheritance | Incremental |
|  | V800R012SPH070 | Inheritance | Incremental |
|  | V800R012SPH060 | Inheritance | Incremental |
|  | V800R012SPH059 | Inheritance | Incremental |
|  | V800R012SPH057 | Inheritance | Incremental |
|  | V800R012SPH055 | Inheritance | Incremental |
|  | V800R012SPH053 | Inheritance | Incremental |
|  | V800R012SPH052 | Inheritance | Incremental |
|  | V800R012SPH039 | Inheritance | Incremental |
|  | V800R012SPH036 | Inheritance | Incremental |
|  | V800R012SPH032 | Inheritance | Incremental |
|  | V800R012SPH031 | Inheritance | Incremental |
|  | V800R012SPH028 | Inheritance | Incremental |
|  | V800R012SPH023 | Inheritance | Incremental |

## Impact of Patch Installation

See the Solution Impact and Precautions for Installing the Patch columns in the NE40E-M2 V800R012SPH330 Issues Resolved.

## Precautions

            - It is acceptable if the patch file is loaded to boards asynchronously.

            - During patch installation or uninstallation, ensure that all boards in use have been registered. If any interface board is starting during patch installation or uninstallation, the patch installation or uninstallation will probably fail on this interface board.

            - Do not install the patch if the board CPU usage exceeds 70%. Otherwise, patch installation may fail.

            - Do not install the patch if the board memory usage exceeds 80%. Otherwise, the board may reset due to a failure to apply for memory. To check the memory and CPU usage, run the display health command in the system view.

[HUAWEI]display health

-------------------------------------------------------------------------------

Slot   CPU        Usage     Memory    Usage(Used/Total)

3     IPU           19%       22%        3473MB/15398MB

-------------------------------------------------------------------------------

            - It is normal that the board CPU usage temporarily rises during patch installation or uninstallation. If the device has a large number of configurations, the board CPU usage may increase to 90% or higher in a short time.

            - The new function provided by the patch cannot be used if the patch is uninstalled. Therefore, to use the new function, do not uninstall the patch. If the patch needs to be upgraded, use the incremental patch loading mode.

            - When loading a patch that contains a YANG model change, ensure that no NETCONF session is in the configuration editing state. Otherwise, an unknown error may occur.

In a scenario in which a user uses Telnet to access a NE40E-M2 through an interface on an interface board, if you want to restart the interface or power off and then power on the interface board or subcard where the interface resides for the installed patch to take effect, ensure that there is a backup Telnet connection to another interface board. If there is no backup Telnet connection, do not restart the interface or power off and then power on the interface board or subcard where the interface resides. Otherwise, the user will fail to access the NE40E-M2.

After the patch is installed and activated, you must restart the board or interface. For detailed procedures, see section 3.3.

## Naming Rules for Patches

Patches are classified as hot patches and cold patches based on the impacts that patches have on user experience when they are activated and validated.

A hot patch (HP) does not affect user experience because it does not cause service interruptions when it is activated and validated.

A cold patch (CP) affects user experience because it causes service interruptions when it is activated and validated.

Naming rules for Accumulated Correction Updates (ACUs) are as follows:

            - For an ACU that is released based on the previous cold ACU, if the current ACU contains patches that would affect user experience when being activated and validated, the current ACU is a cold ACU and named SPCyyy.

            - For an ACU that is released based on the previous cold ACU, if the current ACU does not contain any patches that would affect user experience when being activated and validated, the current ACU is a hot ACU and named SPHyyy.

Naming rules for Emergency Correction Patches (ECPs) are as follows:

            - For an ECP that is released based on the previous ACU, if activating and validating the ECP would not affect user experience, the ECP is named as a hot ECP in the format of HPyyyy; if activating and validating the ECP would affect user experience, the ECP is named as a cold ECP in the format of CPyyyy.

            - The first y in HPyyyy or CPyyyy is fixed at 0, and the subsequent yyy is the same as yyy in SPCyyy or SPHyyy of the previous ACU. Therefore, an ECP is named in the format of HP0yyy or CP0yyy. If a calculated ECP name is the same as that of the previously released ECP, "yyy" in the newly calculated one increases by 1.

For example, after a cold ACU named SPC011 is released, the first hot ECP is named HP0011, and the next hot ECP is named HP0012. After a hot ACU named SPH012 is released, the first cold ECP is named CP0013 because the number 0012 has already been used by HP0012 for the same ACU.

Activating and validating a hot ACU or an ECP released based on the previous cold ACU and a later released hot ACU would not affect user experience. However, activating and validating a hot ACU or an ECP released based on an ACU earlier than the previous cold ACU or on a device that is not equipped with any patches would affect user experience. Therefore, comply with the precautions for activating and validating cold ACUs when performing these operations.

# Resolved Issues

This section describes the issues that are resolved in V800R012 and precautions for installing the patch. For detailed information, see the NE40E-M2 V800R012SPH330 Issues Resolved.

Pay attention to the precautions for installing the patch. The possible causes are as follows: Restart the device, perform an master/slave switchover, power off and then power on the board or subcard, reset the board or subcard, reconfigure the device, or add configurations. Exercise caution when performing this operation.

## How to Obtain Issues Resolved

The website does not support online browsing of Excel files. Therefore, you cannot view the Issues Resolved online. Download and view the list.

- Download the Patch Release Notes Zip package to the local host.

- Decompress the Patch Release Notes and view the Issues Resolved.

# Installing the Patch

## Tool-based Patch Upgrade (Recommended)

Carriers are advised to use uUpgrade in IP Toolkit for patch installation and upgrade. The download path of IP Toolkit and the tool patch is as follows:

https://support.huawei.com/carrier/navi?coltype=software#col=software&path=PBI1-7275726/PBI1-7275757/PBI1-21039046/PBI1-21621186

Enterprise customers are advised to use uUpgrade in eDesk Pro for patch installation and upgrade. The download path of eDesk Pro is as follows:

https://support.huawei.com/enterprise/en/enterprises-common/edesk-pro-pid-23037235/software

Download the latest version and patch of the tool on the linked page for installation.

## Installing the Patch

1. The following patch installation procedure is applicable when the device is to be installed with a patch for the first time or the patch on the device needs to be upgraded to a later version.

2. If this patch is an incremental one of the original patch, directly load this patch. If this patch is not an incremental one of the original patch, unload the original one and then load this patch. (To check whether a patch is an incremental one, see section 1.7 "Relationship Between Patches".)

### Preparing to Install

Before installing the patch, perform the following steps to make necessary preparations.

          - Obtain the NE40E-M2 V800R012SPH330 patch from https://support.huawei.com.

          - Run the display version command on the device to check whether the software version matches the patch version. For details, see section 1.6 Product Version and Required Patches.

<HUAWEI>display version

Huawei Versatile Routing Platform Software 
VRP (R) software, Version 8.xxx (NE40E-M2 V800R012CxxSPCxxx) 
Copyright (C) 2012-2020 Huawei Technologies Co., Ltd.

          - Verify the digital signature of the patch package.

To prevent the software package from being maliciously tampered with during transmission or storage, download the related digital signature file for integrity verification when downloading the software package.

After the software package is downloaded, verify the PGP digital signature of the software package by following the procedure in OpenPGP Signature Verification Guide. If the verification fails, do not use the software package. Contact Huawei technical support.

Before using the software package for installation or upgrade, verify the digital signature of the software package to ensure that the software package is not tampered with.

For carrier customers, visit https://support.huawei.com/carrier/digitalSignatureAction.

- For enterprise customers, visit https://support.huawei.com/enterprise/en/tool/pgp-verify-TL1000000054.

          - Upload the NE40E-M2V800R012SPH330.PAT file to cfcard:/ using FTP in binary mode, after ensuring that the patch name does not conflict with any existing file name on the NE40E-M2.

          - If the slave MPU or SRU exists, copy the patch to the slave MPU or SRU running the following command:

<HUAWEI> copy NE40E-M2V800R012SPH330.PAT slave#cfcard:/

          - Run the display patch-information process verbose | include NO | exclude Running command in the diagnostic view to check the patch unit status. If the following command output is displayed, each patch unit is running properly. If a different command output is displayed, some patch units are not running properly. To troubleshoot this problem, contact Huawei R&D engineers.

[~HUAWEI-diagnose]display patch-information process verbose | include NO | exclude Running

Info: It will take a long time if the content you search is too much or the string you input is too long, you can press CTRL_C to break.

---------------------------------------------------------------------------------------

Slot-id  ProcId   State     PatchType Valid     PatchEffectiveTime            PatchFileName

---------------------------------------------------------------------------------------

---------------------------------------------------------------------------------------

The patch is large, and uploading the patch takes some time. If the CF card has sufficient space, you can upload the patch to the NE40E-M2 before uninstalling existing patches. If the patch name conflicts with an existing file name, modify the patch name before uploading the patch.

----End

### Installing the Patch

Use the patch load NE40E-M2V800R012SPH330.PAT all run command to install the patch. If you use other commands, some issues may fail to be resolved.

Run the patch load NE40E-M2V800R012SPH330.PAT all run command to load, activate, and run the patch.

The system displays the patch installation results.

For example:

<HUAWEI> patch load NE40E-M2V800R012SPH330.PAT all run

Info: Operating, please wait for a moment...done.

Info: Succeeded in running the patch.

## Verifying Patch Installation

- Run the display patch-information process verbose | include NO | exclude Running command in the diagnostic view to verify that each patch unit is running properly. If the following command output is displayed, each patch unit is running properly. If a different command output is displayed, some patch units are not running properly. To troubleshoot this problem, contact Huawei R&D engineers.

[~HUAWEI-diagnose]display patch-information process verbose | include NO | exclude Running

Info: It will take a long time if the content you search is too much or the string you input is too long, you can press CTRL_C to break.

---------------------------------------------------------------------------------------

Slot-id  ProcId   State     PatchType Valid     PatchEffectiveTime            PatchFileName

---------------------------------------------------------------------------------------

---------------------------------------------------------------------------------------

### NE40E-M2 V800R012C10SPC300

<HUAWEI> display patch-information

Patch Package Name    :cfcard:/NE40E-M2V800R012SPH330-C10SPC300.PAT

Patch Package Version :V800R012SPH330

Patch Package State   :Running

Patch Package Run Time:2023-04-07 10:57:30

## Follow-up Procedures After Patch Installation

After patch installation, check whether issues described in the NE40E-M2 V800R012SPH330 Issues Resolved.xlsx delivered with the patch release notes exist on the device. If any issues exist, perform operations listed in the corresponding "Operation Type for Patch Validation" and "Operation Type for Service Restoration " in the NE40E-M2 V800R012SPH330 Issues Resolved.xlsx. The possible causes are as follows: Restart the device, perform an master/slave switchover, power off and then power on the board or subcard, reset the board or subcard, reconfigure the device, or add configurations. Exercise caution when performing this operation.

# Rollback Procedure

## How to Roll Back to the Previous State

If patch installation fails and a rollback is necessary, perform the following steps to roll back to the previous state.

          - Run the display patch-information command in the user view to check the patch running status.

          - Run the patch delete all command in the user view to delete all patches.

          - Run the display patch-information command in the user view to check whether the patches are still running. Ensure that the patches are not running.

----End

When the patch is being uninstalled, do not run any commands. Otherwise, services may become abnormal or the master main control board may be reset. After the patch is uninstalled, you are advised to log out all users who logged in through terminal tools.

# Solutions to Common Problems During Patch Installation

## A Message "Error: The system file should be in root directory." Is Displayed After the patch load all run Command Is Run to Load a Patch File.

      - Symptom

<HUAWEI>patch load $_install_mod/NE40E-M2V800R012SPHxxx.PAT all run

Info: Operating, please wait for a moment...

Error: The system file should be in root directory.

      - Solution

This problem occurs if a patch file to be loaded is not stored in the root directory of the master MPU. Therefore, to resolve this problem, copy the patch file to the root directory on the CF card before running the patch load NE40E-M2V800R012SPHxxx.PAT all run command in the user view.

## A Message "Error: The command is invalid in the current patch status." Is Displayed After the patch load all run Command Is Run to Load a Patch File.

      - Symptom

<HUAWEI>patch load NE40E-M2V800R012SPHxxx.PAT all run

Info: Operating, please wait for a moment...

Error: The command is invalid in the current patch status.

      - Solution

This problem occurs if a patch has been installed and the patch is not in the running state. Therefore, to resolve this problem, run the display patch-information command in the user view to check whether a patch has been installed and whether the patch is in the running state.

<HUAWEI>display patch-information

Patch Package Name    :cfcard:/NE40E-M2V800R012SPHxxx.PAT

Patch Package Version :V800R012SPHxxx

Patch Package State   :Deactive

In this example, a patch has been loaded and the patch is not in the running state.

In this case, before loading the new patch, run the patch load NE40E-M2V800R012SPHxxx.PAT all run command in the user view to have the existing patch enter the running state or run the patch delete all command to delete the existing patch.

## A Message "Error: The patch is incompatible with the existing patch." Is Displayed After the patch load all run Command Is Run to Load a Patch File.

      - Symptom

<HUAWEI>patch load NE40E-M2V800R012SPHxxx.PAT all run

Info: Operating, please wait for a moment...

Error: The patch is incompatible with the existing patch.

      - Solution

This problem occurs if the patch in the file to be loaded is not an incremental one of the existing patch on the device. Therefore, to resolve this problem, run the patch delete all command in the user view to delete the existing patch before loading the new patch.

## How Can I Set a Patch as That to Be Loaded at the Next Startup?

- A: Run the following commands on the master and slave main control boards:

<HUAWEI> startup patch cfcard:/NE40E-M2V800R012SPHxxx.PAT all

Info: Operating, please wait for a moment.....done.

Info: Succeeded in setting startup the patch.

- Vulnerabilities Fixed

You can query vulnerabilities at the National Vulnerability Database (NVD) website (https://web.nvd.nist.gov/view/vuln/search) by CVE ID.

  - V800R012C10SPC300

- Table A-2-1: Fixed open-source and third-party software vulnerabilities

| Software Name | Software Version | CVE ID | CVSS | Vulnerability Description | Vulnerability-specific Patch Version |
| --- | --- | --- | --- | --- | --- |
| libyang | 1.0.130 | CVE-2021-28902 | 7.5 | In function read_yin_container() in libyang <= v1.0.225, it doesn't check whether the value of retval->ext[r] is NULL. In some cases, it can be NULL, which leads to the operation of retval->ext[r]->flags that results in a crash. | V800R012SPH055 |
| libyang | 1.0.130 | CVE-2021-28903 | 7.5 | A stack overflow in libyang <= v1.0.225 can cause a denial of service through function lyxml_parse_mem(). lyxml_parse_elem() function will be called recursively, which will consume stack space and lead to crash. | V800R012SPH055 |
| libyang | 1.0.130 | CVE-2021-28904 | 7.5 | In function ext_get_plugin() in libyang <= v1.0.225, it doesn't check whether the value of revision is NULL. If revision is NULL, the operation of strcmp(revision, ext_plugins[u].revision) will lead to a crash. | V800R012SPH055 |
| libyang | 1.0.130 | CVE-2021-28905 | 7.5 | In function lys_node_free() in libyang <= v1.0.225, it asserts that the value of node->module can't be NULL. But in some cases, node->module can be null, which triggers a reachable assertion (CWE-617). | V800R012SPH055 |
| libyang | 1.0.130 | CVE-2021-28906 | 7.5 | In function read_yin_leaf() in libyang <= v1.0.225, it doesn't check whether the value of retval->ext[r] is NULL. In some cases, it can be NULL, which leads to the operation of retval->ext[r]->flags that results in a crash. | V800R012SPH055 |
| OpenSSL | 1.1.1-1.1.1j | CVE-2021-3449 | 4.9 | When TLSv1.2 and renegotiation (enabled by default) are enabled on the server, if the extension field signature_algorithms exists before renegotiation but does not exist after renegotiation (and the extension field signature_algorithms_cert does not exist after renegotiation either), a NULL pointer may be referenced, causing DoS attacks. | V800R012SPH059 |

- Table A-2-2: Fixed vulnerabilities in Huawei-developed code (including products and integrated platforms/components/libraries)

| Vulnerability ID | CVE ID | CVSS | Vulnerability Description | Solution Version |
| --- | --- | --- | --- | --- |
| HWPSIRT-2020-70186 | N/A | 5.3 | When IS-IS receives a TLV with the Length field being 0, the main control board restarts unexpectedly. | V800R012SPH028 |
| HWPSIRT-2021-64225 | N/A | 5.9 | When a Linux client logs in to a device using public key authentication through SSH, the main control board of the device may be abnormal. | V800R012SPH028 |
| HWPSIRT-2021-78991 | N/A | 6.7 | The Python sandbox of the OPS feature has sandbox escape and arbitrary code execution vulnerabilities. | V800R012SPH036 |
| HWPSIRT-2021-32380 | N/A | 6.7 | A user uses the OPS function to execute a Python script to tamper with a device file. | V800R012SPH059 |
| HWPSIRT-2021-22914/HWPSIRT-2021-37794 | N/A | 8.2 | The import keyword can be used to import the .pyc file of malicious bytecode. In this way, LOAD_FAST and LOAD_CONST can be used to implement Python sandbox escape and execute malicious C code or assemble binary code. | V800R012SPH070 |
| HWPSIRT-2022-46709 | CVE-2022-0778 | 7.5 | The BN_mod_sqrt() function, which computes a modular square root, contains a bug that can cause it to loop forever for non-prime moduli. Internally this function is used when parsing certificates that contain elliptic curve public keys in compressed form or explicit elliptic curve parameters with a base point encoded in compressed form. It is possible to trigger the infinite loop by crafting a certificate that has invalid explicit curve parameters. Since certificate parsing happens prior to verification of the certificate signature, any process that parses an externally supplied certificate may thus be subject to a denial of service attack. The infinite loop can also be reached when parsing crafted private keys as they can contain explicit elliptic curve parameters. Thus vulnerable situations include: - TLS clients consuming server certificates - TLS servers consuming client certificates - Hosting providers taking certificates or private keys from customers - Certificate authorities parsing certification requests from subscribers - Anything else which parses ASN.1 elliptic curve parameters Also any other applications that use the BN_mod_sqrt() where the attacker can control the parameter values are vulnerable to this DoS issue. In the OpenSSL 1.0.2 version the public key is not parsed during initial parsing of the certificate which makes it slightly harder to trigger the infinite loop. However any operation which requires the public key from the certificate will trigger the infinite loop. In particular the attacker can use a self-signed certificate to trigger the loop during verification of the certificate signature. This issue affects OpenSSL versions 1.0.2, 1.1.1 and 3.0. It was addressed in the releases of 1.1.1n and 3.0.2 on the 15th March 2022. Fixed in OpenSSL 3.0.2 (Affected 3.0.0,3.0.1). Fixed in OpenSSL 1.1.1n (Affected 1.1.1-1.1.1m). Fixed in OpenSSL 1.0.2zd (Affected 1.0.2-1.0.2zc) | V800R012SPH073 |
| HWPSIRT-2022-88327 | CVE-2022-0391 | 7.5 | The urlparse function in the python urllib.parse module does not filter special characters such as \r\n when splitting paths, causing URL injection risks. | V800R012SPH073 |
| HWPSIRT-2021-15235 | NA | 5.8 | The PCEP function of datacom products has a memory overwriting vulnerability. Attackers can attack devices using crafted packets, resulting in memory overwriting. This affects device availability. | V800R012SPH2b0 |